# AAA

Lab Goal:

- Installation of CA Certificate Authority.

- Installation and configuration of FreeRADIUS AAA server.

- Configuration of WPA2 Enterprise Wi-Fi network.

- Configuration of CISCO network devices to use AAA.

Lab Requirements:

- Netgate FW appliance.

- Debian client.

- Internet Connection.

Table of Contents

AAA	1

3

FreeRADIUS	3

History	3

Technical Summary	3

Task 1 – Configuration of CA	4

Step 1: Click on System > Certificates.	4

Step 2: Configure CA	4

Task 2 – Configuring AAA Server certificates	6

Step 1: Click on Certificates Tab	6

Step 2: Configure Server Certificate	6

Task 3 – Install Free Radius AAA server	7

Step 1: Open System > Packet Manager	7

Step 2: Select Available Packages Tab.	8

Step 2: Create Wireless User Account in FreeRADIUS.	9

Step 3: Create device account for Wi-Fi Access Point	9

Task 4 – Create an interface.	10

Step 1:  Create Authentication interface.	10

Task 5 – Configure your Access Point	12

Step 1:  Connect your AP.	12

Step 2 -Switch mode to AP	12

Step3: Changing Network Mode	13

Step 4: Create SSID – name of Wi-Fi network	14

Step 5: Disable DHCP as we are running one on Netgate.	14

Step 6: Configure Wireless Security	14

Task 6 – Connect to new Wi-Fi network	15

Task 7 – Configure FreeRADIUS for Cisco devices.	15

Step 1:   Create AAA Cisco Admin account.	15

Step 2:  Create Device Account for SW1 and SW2	16

Task 8 – Configure SW1 and SW2 to use AAA on FreeRADIUS.	17

Step 1:  SW1 AAA configuration	17

Step 2:  Configure VTY Lines	17

Step 3:  Connect to SW1 using Debian client.	18

Step 4:  Configure SW2.	19

Step 2:  Configure VTY Lines	19

Step 3:  Connect to SW2 using Debian client.	19

Lab Challenge.	20

References	20

## FreeRADIUS

FreeRADIUS is an open-source Remote Authentication Dial-In User Service (RADIUS) server widely used for network authentication, authorization, and accounting. Here’s a brief technical summary along with a bit of its history:

### History

- Origins: FreeRADIUS was initially developed in 1999 as a fork of the Cistron RADIUS server by Alan DeKok and Miquel van Smoorenburg. The goal was to create a more robust and extensible RADIUS server.

- Development: Over the years, FreeRADIUS has become the most popular and widely deployed RADIUS server, used by millions of users and organizations globally.

- Community and Support: FreeRADIUS is maintained by a community of developers and is supported by the FreeRADIUS Foundation.

### Technical Summary

- Authentication Protocols: FreeRADIUS supports a wide range of authentication protocols, including PAP, CHAP, MS-CHAP, MS-CHAPv2, EAP (TLS, TTLS, PEAP, LEAP, etc.), and others, making it suitable for various applications.

- Authorization: It can be integrated with various backends for authorization, such as SQL databases (MySQL, PostgreSQL, SQLite), LDAP directories, and even flat files.

- Accounting: FreeRADIUS provides comprehensive accounting features, logging all aspects of user sessions, which is crucial for auditing and billing purposes.

- Extensibility: The server is highly modular, allowing for customization and extension through modules. Modules can handle authentication, authorization, and accounting functions, or integrate with external systems.

- Performance: Known for its high performance, FreeRADIUS can handle thousands of authentication requests per second, making it suitable for large-scale deployments.

- Compatibility: FreeRADIUS works with a variety of network devices (routers, switches, access points) from numerous vendors, ensuring broad compatibility.

- Security: It supports advanced security features, including IPsec, TLS encryption, and the use of certificates for mutual authentication.

- Configuration: The configuration is managed through text-based files, providing flexibility and ease of automation. Policies and rules can be finely tuned to meet specific network requirements.

- Community and Documentation: A large user base and comprehensive documentation provide a wealth of resources for troubleshooting and extending functionality.

FreeRADIUS remains a critical component for organizations requiring secure, scalable, and reliable network authentication solutions, continuing to evolve with the needs of modern networking environments.

## Task 1 – Configuration of CA

In this task we will be configuring Certificate authority, and we will generate new Certificates we will use for our Wi-Fi WPA2 enterprise network.

### Step 1: Click on System > Certificates.

Click on Add

### Step 2: Configure CA

Configure CA as per specification:

  - Descriptive Name: CybertechCA

  - Method: Create an internal Certificate Authority

  - Randomize Serial: Checked

  - Country Code: AU

  - State or Province: Tasmania

  - City: Hobart

  - Organization: Cybertech

  - Organizational Unit: SOC

  - Click on save.

Expected screenshot:

## Task 2 – Configuring AAA Server certificates

Open Server Certificate Tab

### Step 1: Click on Certificates Tab

### Step 2: Configure Server Certificate

Click on add/sign at bottom of page.

Configure Server Certificate as per specification:

- Descriptive Name: AAA Cert

- Lifetime: 365

- Common Name: aaa.cybertech.lab

- Certificate Type: Server Certificate

Change to Server Certificate from drop down list.

Screenshot of selection

- Save.

## Task 3 – Install Free Radius AAA server

### Step 1: Open System > Packet Manager

### Step 2: Select Available Packages Tab.

Search for radius (a) and click search button (b).

Add FreeRADIUS (a) by clicking on install button (b) on right.

Confirm

Expected screenshot:

### Step 2: Create Wireless User Account in FreeRADIUS.

Click on Service > FreeRADIUS

Create a User by clicking on add.

- Username: wifiuser

- Password: wifipass

- Password Encryption: Cleartext-Password

Note: As this is lab environment we will use clear text password, however we have learned that it is not best practice.

Research what is needed for non-clear text pass to be integrated.

- Save

Expected screenshot:

### Step 3: Create device account for Wi-Fi Access Point

Click on NAS/Clients and click on Add button.

Change IP accordingly to your group. If IP is used advise, please.

- Client IP Address: 10.1.1.254

- Client Version: IPv4

- Client Shortname: wifi_ap

- Client Shared Secret: aaasapsecret

- Save

Expected screenshot:

## Task 4 – Create an interface.

### Step 1:  Create Authentication interface.

This interface will be used by FreeRADIUS to listen for incoming Authentication requests.

Click on Add.

- Interface IP Address: *

- Port: 1812

- Interface Type: Authentication

- IP Version: IPv4

- Description: Auth Interface

- Save

Expected screenshot:

Step 2:  Configure EAP.

Select EAP Tab.

Default EAP Type: MSHAPv2

- SSL CA Certificate: CybertechCA

- SSL Revocation List: none

- SSL Server Certificate: AAA Cert

- Save.

Note: You will not see new page.

## Task 5 – Configure your Access Point

### Step 1:  Connect your AP.

- Power up AP at your desk.

- Start Debian VM

- Configure Debian Network Adapter in VMware to be bridged.

- Configure Ethernet Adapter to DHCP.

- Connect PC Ethernet cable to LAN port of AP

- Access web interface (find out your IP and what is DHCP address and access it as Web interface will be on same host)

- Enter default credentials for model. You will need to research a bit

### Step 2 -Switch mode to AP

- Select Operation mode

- Select Access Point

- Save

AP will reboot

.

### Step3: Changing Network Mode

- Click on Network

- LAN

- Change to static IP

- Assign correct IP based on your Group.

- Save

AP will reboot.

### Step 4: Create SSID – name of Wi-Fi network

- Under Wireless Tab

- Wireless: Enable

- Wireless Network Name: GROUP1

Name is example use your group name.

- Save

### Step 5: Disable DHCP as we are running one on Netgate.

- Under DHCP Tab

- DHCP Server: Disable

### Step 6: Configure Wireless Security

Go to Wireless security and configure WPA2 Enterprise

Select

pfsense ip

aaasapsecret

Save settings.

## Task 6 – Connect to new Wi-Fi network

Use TasTAFE laptops to connect.

Note: There will be a warning with CA as it is self-signed.

## Task 7 – Configure FreeRADIUS for Cisco devices.

### Step 1:   Create AAA Cisco Admin account.

In this step we need to create CISCO AAA Admin account.

To achieve this, you can follow same steps as described in previous Task.

- Username: aaaciscoadmin

- Password: aaaciscopass

- Password Encryption: Cleartext-Password

- Save

Expected screenshot.

### Step 2:  Create Device Account for SW1 and SW2

In this step we need to create device account same way we did in previous Task for Wi-Fi AP.

Link

Example for G2SW1

Change IP accordingly to your group. Client IP Address: 10.1.2.11

- Client Version: IPv4

- Client Shortname: G2SW1

- Client Shared Secret: aaasw1secret

- Save

Example for GSW2

Change IP accordingly to your group.

- Client IP Address: 10.1.2.12

- Client Version: IPv4

- Client Shortname: G2SW2

- Client Shared Secret: aaasw2secret

- Save

### Task 8 – Configure SW1 and SW2 to use AAA on FreeRADIUS.

### Step 1:  SW1 AAA configuration

NOTE: This is example use appropriate names and IPv4 addresses

- Enable AAA

G2SW1(config)#aaa new-model

- Crete default login

G2SW1(config)#aaa authentication login default group radius local

- Configure AAA Server connection

G2SW1(config)#radius server pfsense

G2SW1(config-radius-server)#address ipv4 10.1.2.1 auth-port 1812

G2SW1(config-radius-server)#key aaassw1secret

G2SW1(config-radius-server)#exit

G2SW1(config)#

- Test connectivity using new AAA user.

SSH to SW1

### Step 2:  Configure VTY Lines

G2SW1(config)#line vty 0 15

G2SW1(config-line)#login authentication default

### Step 3:  Connect to SW1 using Debian client.

NOTE: If you get error

You will need to edit ssh client to accept outdated cyphers offered by CISCO

- Open ssh client configuration file.

sudo nano /etc/ssh/ssh_config

- Locate the line ‘ #   Ciphers aes128-ctr,aes192-ctr,aes256-ctr,aes128-cbc,3des-cbc’ and remove the Hash/Pound sight from the beginning.

- Locate the line ‘#   MACs hmac-md5,hmac-sha1,umac-64@openssh.com,hmac-ripemd160′ and remove the Hash/Pound # sight from the beginning,

- Then paste the following on the end;"

HostkeyAlgorithms ssh-dss,ssh-rsa

KexAlgorithms +diffie-hellman-group1-sha1,diffie-hellman-group14-sha1

### Step 4:  Configure SW2.

- Enable AAA

G2SW2(config)#aaa new-model

- Crete default login

G2SW2(config)#aaa authentication login default group radius local

- Configure AAA Server connection

G2SW2(config)#radius server pfsense

G2SW2(config-radius-server)#address ipv4 10.1.2.1 auth-port 1812

G2SW2(config-radius-server)#key aaassw1secret

G2SW2(config-radius-server)#exit

G2SW2(config)#

- Test connectivity using new AAA user.

SSH to SW2

### Step 2:  Configure VTY Lines

G2SW1(config)#line vty 0 15

G2SW1(config-line)#login authentication default

### Step 3:  Connect to SW2 using Debian client.

NOTE: If you get error refer to previous Step how to solve it.

Expected screenshot:

### Lab Challenge.

As this Lab is challenging enough there will not be challenges this time.

### References

Official NetGate Documentation.

RADIUS Authentication Servers | pfSense Documentation (netgate.com)

Official CISCO documentation