### Basic Networking configuration and securing local and remote access.

Table of Contents

Task 1: Basic sw1 Configuration:	1

Task 2 : Basic R1 Configuration	5

Task 3: Basic SW2 Configuration:	6

Task 4 : Configure PC1 and test connectivity between PC0 and PC1	6

`

Goal for the Lab:

- Implement initial switch and router configuration based on specification.

- Secure local access (console lines, AUX).

- Secure remote access to all devices (VTY Lines).

| Device Name | Port | IPV4 | Gateway |
| --- | --- | --- | --- |
| PC0 | Fa0 | 192.168.1.100/24 | 192.168.1.1/24 |
| PC1 | Fa0 | 10.1.1.100/24 | 10.1.1.1/24 |
| SW1 | Fa0/1 | N/A | N/A |
|  | Fa0/2 | N/A | N/A |
|  | VlAN1 | 192.168.1.200/24 | 192.168.1.1/24 |
| SW2 | Fa0/1 | N/A | N/A |
|  | Fa0/2 | N/A | N/A |
|  | VlAN1 | 10.1.1.200/24 | 10.1.1.1/24 |
| R1 | Gig0/0/0 | 192.168.1.1/24 | N/A |
|  | Gig0/0/1 | 10.1.1.1/24 | N/A |

## Task 1: Basic sw1 Configuration:

- Connect to Console port.

- Enter Priv Exec Mode.

enable

- Enter configuration mode.

configure terminal

- Set up hostname

hostname sw1

- Secure Priv Exec access

enable secret ciscoen

- Assign VLAN 1 IP

int vlan 1

ip add 192.168.1.200 255.255.255.0

no shut

- Check ip configuration:

do show ip interfaces brief

You should see same output as in screenshot.

- Configure PC0 as per specifications and ping SW1 VLAN 1 interface IP

- Disable DNS domain lookup.

no ip domain-lookup

- Enable service encryption that will encrypt all plain text passwords

service password-encryption

- Configure SSH server

- Configure Domain name

ip domain-name sw1.local

- Generate RSA keys

crypto key generate rsa general-keys modulus 2048

- Set up SSH to V2

ip ssh version 2

- Configure inactivity timer to 180 seconds

ip ssh time-out 180

- Configure maximum unsuccessful login attempts to 2.

ip ssh authentication-retries 2

- Create Local username and password -  privilege 15 is CISCO way to assign user to admin group.

username student privilege 15 password cisco

- Configure Console line to use local database for authentication:

line con 0

login local

- Configure VTY lines to use local database for authentication and enforce ssh only to be used.

line vty 0 15

login local

transport input ssh

- Test Console lines

Exit until you reach login screen

If all is configured correctly you should see prompt for username and password.

- Connect from PC0 using ssh

ssh -l student 192.168.1.200

- Configure sw1 default gateway

ip default-gateway 192.168.1.1

- Save configuration:

copy running-config startup-config

## Task 2 : Basic R1 Configuration

- Set Hostname to R1

- Secure Priv Exec access -  enable secret ciscoen

- Disable dns domain lookup

- Enable service encryption that will encrypt all plain text passwords

- Configure SSH server

- Configure Domain name - r1.local

- Generate RSA keys – length 2048

- Set up SSH to V2

- Configure inactivity timer to 180 seconds

- Configure maximum unsuccessful login attempts to 2

- Create Local username and password - username student privilege 15 password cisco

- Assign interface Gig0/0/0 IP to 192.168.1.1/24

- Assign interface Gig0/0/0 IP to 10.1.1.1/24

- Check Interface status:

- Configure Console line to use local database for authentication:

- Configure VTY lines to use local database for authentication – enable ssh access only

- In Configuration mode limit number of unsuccessful attempts

Note additional options.

login block-for 300 attempts 3 within 4

this command will block user for 300 seconds if there has been 3 unsuccessful login attempt within 4 seconds

- Test Connectivity from PC0

- Save configuration:

## Task 3: Basic SW2 Configuration:

1)	Set Hostname to sw2

2)	Secure Priv Exec access -  enable secret ciscoen

3)	Disable dns domain lookup

4)	Enable service encryption that will encrypt all plain text passwords

5)	Configure SSH server

6)	Configure Domain name - r1.local

7)	Generate RSA keys – length 2048

8)	Set up SSH to V2

9)	Configure inactivity timer to 180 seconds

10)	Configure maximum unsuccessful login attempts to 2

11)	Create Local username and password - username student privilege 15 password cisco

12)         Assign VLAN 1 IP - 10.1.1.200/24

13)	 Configure default gateway -  10.1.1.200

## Task 4: Configure PC1 and test connectivity between PC0 and PC1

- SSH from PC1 to sw1

- SSH from PC1 to r1

- SSH from PC1 to sw2

## Troubleshooting commands:

Connectivity:

ping 10.1.1.1

traceroute 10.1.1.1

telnet 10.1.1.1

show interface g0/0/0

show ip interface g0/0/0

show ip interface brief

SSH:

show ip ssh - verify the optional SSH command settings.

show ssh - verify the status of the client connections.